Ch 8 Homework Solutions:  Quality Control

Problem 8-1

a.  Cpk=minUTL−X3σ,X−LTL3σ=min1.01−1.0023(.003),1.002−.993(.003)=min.889,1.333

= .889

b.   The process is capable of producing the desired quality, but at present it is not capable

due to the process center.  The process center should be adjusted.

Problem 8-2

a.  	Ten defectives were found in 10 samples of size 15.

P=1010(15)= .067

Sp=P(1−P)n=.067(1−.067)15= .0645

UCL = P + 1.96Sp =  .067 + 1.96(.0645) = .194

UCL = P - 1.96Sp =  .067 - 1.96(.0645)  = -.060 → zero

b.	Stop the process and look for the special cause, it is out of statistical control.

Problem 8-3

| Sample | 1 | 2 | 3 | 4 | mean | range |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 1010 | 991 | 985 | 986 | 993.00 | 25 |
| 2 | 995 | 996 | 1009 | 994 | 998.50 | 15 |
| 3 | 990 | 1003 | 1015 | 1008 | 1004.00 | 25 |
| 4 | 1015 | 1020 | 1009 | 998 | 1010.50 | 22 |
| 5 | 1013 | 1019 | 1005 | 993 | 1007.50 | 26 |
| 6 | 994 | 1001 | 994 | 1005 | 998.50 | 11 |
| 7 | 989 | 992 | 982 | 1020 | 995.75 | 38 |
| 8 | 1001 | 986 | 996 | 996 | 994.75 | 15 |
| 9 | 1006 | 989 | 1005 | 1007 | 1001.75 | 18 |
| 10 | 992 | 1007 | 1006 | 979 | 996.00 | 28 |
| 11 | 996 | 1006 | 997 | 989 | 997.00 | 17 |
| 12 | 1019 | 996 | 991 | 1010 | 1004.00 | 28 |
| 13 | 981 | 991 | 989 | 1003 | 991.00 | 22 |
| 14 | 999 | 993 | 988 | 984 | 991.00 | 15 |
| 15 | 1013 | 1002 | 1005 | 992 | 1003.00 | 21 |

X= 999.1, R= 21.733

Control limits for X-bar chart:

UCL, LCL = X+A2R, = 999.1 + .73(21.733) = 1014.965, 983.235

Control limits for R chart:

UCL  = D4R = 2.28(21.7333) = 49.551

LCL = D3R = 0(21.7333) = 0.00

The process is in statistical control.